| Pump | Service | Power (kW) | Flow (gpm@4000 rpm) |
| --- | --- | --- | --- |
| Hilborn BL234 | Very small displacement engines (typically one to four cylinders, for example 108ci Offenhauser or 152ci Chev), running on petrol or methanol. | - | ? |
| McGee “round head 6-bolt” | Short pump designed for Repco head. Not useable. |  | Could not get suction |
| McGee “hex head” | My pump. Flow wakes up at 4000 rpm. Not useable. |  | 0.35@3500rpm |
| McGee “hex head” | My pump, base is multi-hole drilled. | Assume 30% return flow. 160HP on methanol | 1.14 |
| Required flow for FED | Safe methanol tune. 2lb/hr/hp@220hp = 7.33lb.min = 1.11 gpm. This has NO return flow to tank… add 30% or so to get minimum pump size around 1.6 gpm |  | 1.6 |
| Hilborn PG150 #00 | Small engines such as motorcycles or four cylinder engines, running on petrol or methanol. | 200 | 1.7 (0.67@ 1899 RPM) |
| BL420 | My pump. Very small displacement engines (typically one to four cylinders, for example 108ci Offenhauser or 152ci Chev), running on petrol or methanol. | Assume 30% return flow. 380HP on methanol | 2.75 |
| PG150C #0 | My pump, came with angle drive | Assume 30% return flow. 480HP on methanol | 3.4 |
| Hilborn PG150 #0 | Medium size engines, 200″ to 360″, running petrol or methanol. Use with carburettors or small turbo engines. | 400 | 3.7 |
| Hilborn PG150 #S0 | Medium size engines, 360-410ci running petrol, methanol, or small amounts of nitromethane. | - | 4.0 |
| Enderle 80A-0 |  |  | 4.5 |
| Hilborn PG150 #1/2 | Same as #S0, just a higher pump volume. | 500 | 5.0 (2.2@ 1899 RPM) |
| Enderle 80A-0.5 |  |  | 5.5 |
| Hilborn PG150 #1 | Large engines on petrol and small amounts of nitromethane. Also for blown engines on petrol, methanol, and small amounts of nitromethane. | 700 | 6.7-7.0 (2.8 @1899 RPM) |
| Enderle 80A-1 |  |  | 7.1 |
| Enderle DSR |  |  | 7.5 |
| Enderle 600 |  |  | 9.2 |
| Enderle 760 |  |  | 11.0 - 11.4 |
| Hilborn PG175 #2 | Large engines with high percentages of nitromethane and blown engines on methanol. | 1300 | 12.2 |
| Enderle 110 |  |  | 12.2 - 13.4 |
| Enderle 990 |  |  | 15.7 |
| Hilborn PG175 #4 | Very large engines, high percentages of nitromethane, and blown engines on nitromethane. | - | 17 |
| Enderle 1100 | 7-tooth idle gear, 1.350-inch OD, 1.100-inch depth |  | 17.2 |
| Enderle 1200 |  |  | 18.8 |
| Enderle 1270 |  |  | 20.2 |
| Enderle 1380 |  |  | 21 |
| Hilborn PG250 #5 | Largest pump volume. Very large engines on nitromethane. Also for turbo or supercharged engines. | - |  |